New Networx Lock Profile Support

 Two double sided lock profiles; DL6300 and PDL6300 are available in the New Lock(s) Profile option.

New Architech Series Support

 Networx Architech Keypads with Prox Card Readers are available in the New Lock(s) Profile option; click on the Lock Type pull down list to select either N90 (cylindrical) or N95 (mortise) Architech lock. Next use the Select Reader Type pull down list to select either Reader Type 7 for  Keypad Prox (125kHz) or Reader Type 8 for Keypad Smart Prox (iClass)

 Networx Architech locks with flashing Emergency Lockout Indicator are supported.

The Architech locks with surface mounted electronics and a transparent (white) Prog/Pass button will have a red flashing LED in the button that is activated during all lockdown conditions.

Use the New Lock(s) Profile option; then click on the Lock Type pull down list; then select either N90 or N95 to access the Select Style pull down list; then select either Style L for Surface Mount Electronics with Lockdown Indicator or Style M for Surface Mount Electronics with Bluetooth and Lockdown Indicator.

 The Architect New Lock Profile screen includes clear descriptions for all options:

When adding an Architech series New Lock Profile; The Select Style pull down menu will display the following descriptions: I = Electronic Insert, J = Electronic Insert with Bluetooth, S = Surface Mount Electronics, L = Surface Mount Electronics with Lockdown Indicator, T = Surface Mount Electronics with Bluetooth, M = Surface Mount Electronics with Bluetooth and Lockdown Indicator.

The Select Reader Type pull down menu will display the following descriptions:

1 = Prox Oval 125kHz, 2 = Prox Square 125kHz, 3 = Swipe Oval, 4 = Swipe Square, 7 = Keypad Prox, 8 = Keypad Smart Prox, A = Smart Oval 13.56MHz, B = Smart Square 13.56MHz

 The Architect series Toggle Mode function is supported:

Located in the Features screen, Functions tab; Select Enable Toggle Mode (Manual Relock) Intended to help avoid accidental lockouts, it works very similar to the "residency" feature on the DL4000 series.

New Bluetooth Capabilities

 Import Bluetooth Credentials:

From the Global screen, DL-Windows can create and send Bluetooth user credentials to any supporting lock and then send a text message to any phone. The text massage allows the phone user to import the Bluetooth credential making that phone a valid user of the lock.

(The iLock phone app is required on the phone)

 Bluetooth User Expiration Date:

From the Global screen, DL-Windows can send a pre-selected date on which to disable a specific Bluetooth user. (Not applicable to locally added Bluetooth phones)

 Bluetooth Schedules:

The Bluetooth ON and Bluetooth OFF schedule events can be set up to enable or disable the Bluetooth radio in the lock. This provides an easy method to limit all Bluetooth user access for any Bluetooth lock on the system. (All locally programmed Bluetooth phones and imported Bluetooth users will be affected)

New Search Options

 User Categories and Search By User Category is supported in v5.5.3:

The "User Information" section located within the Global screen includes the option to assign User Categories to some or all of the basic users in an account. A pull down list includes many  predefined categories. A "User Category" configuration screen allows you to edit any of the predefined category names and add new categories as needed.

The option to search by "User Category" is now part of the user search menu.

 The "Custom Field" data is searchable in v5.5.3.

The user Search section located under the "User list" includes the option to search by Custom Field.

 Account or Lock Description Search capabilities:

The Search field located on the bottom left of the main screen will allow you to Search the entire application by either Account Description or Lock Description:

Enhanced Emergency Lockdown Functions

 The "Exit Only" limited Emergency User configuration is supported:

The user option previously called: Add/Remove Emergency User(s) has been changed to Lockdown User (with additional sub-menu to configure the emergency users lockdown limits)

Step 1: Create a limited, Exit Only, lockdown user:

Open the Global screen; then select one or more users from the user list; then right click the selected user(s); then mouse over Lockdown User; then select Add/Remove Emergency User. (this option will allow the user to initiate two of the three lockdown commands, Emergency Lockdown and Emergency Passage, this user cannot initiate the Emergency Return to Normal command).

Step 2: (optional) To enable the Emergency Return to Normal command for any lockdown user:

Open the Global screen; then select one or more lockdown users from the user list; then right click the selected user(s); then mouse over Lockdown User; then select Enable/Disable as Reset Lockdown User.

 The Main Screen's flashing red and yellow Emergency Lockdown indicator bar will automatically update. This feature will verify that the DL-Windows display will quickly reflect any changes to the systems lockdown status in the event that emergency commands are initiated from individual locks. (The update occurs each time the gateway screen is opened)

 The Enable Emergency Indicator setting is available For Networx lock profiles ETPLN and ETPLNiRx the "Enable Emergency Indicator" check box is located in the Features screen. First open the Features screen, then click on the Emergency tab.

New Schedule Events and Functions

 Custom Holiday Time Zones and Schedule events are supported:

Open the Schedule screen; then to set up a Holiday schedule; first click the Edit button; then click on the Holiday Time Zone check box; then rename the time zone (recommended); then click the Save button. Next configure the Holiday Start On and Holiday End On dates in the Schedule Entry section of the Schedule screen.

 A Power Saving Mode schedule event is supported:

Networx Narrow Style & Double Sided Lock profiles have a Power Saving Mode schedule.

This feature is intended to maximize battery life.

 All Time Zone names are customizable:

Within the Schedule screen, any of the default Time Zone names can be changed by clicking the Edit button; then entering the new name for the individual Time Zone; then click on the Save button.

New Convenience Features

 An Enable Users / Disable Users to All Locks or All Accounts is supported:

Open the Global screen; then select one or more users from the user list; then right click the selected user(s); then mouse over Enable Users; then select any of the following options from the sub-menu: Enable Users to All Accounts; Enable Users to Accounts (use sub-menu to select individual accounts); Enable Users to All Locks; Enable Users to Locks (use sub-menu to select individual locks).

The Disable Users option is similar.

 The user name Sort feature has been modified:

Located in the Global screen (above the user list), Select any of the available sorting options and that specific option will remain in effect indefinitely. The setting will no longer reset to the default option each time the window or application is closed.

 A User Audit Trail, which records DL-Windows actions, is supported:

Located in the File drop down menu; select Audit Trail to view the following:

Logged at (date/time); Location (lock profile name); Action; Operator; Description.

 The firmware version of all Networx locks is listed on the lock list tree:

The currently installed firmware version number of Networx locks is displayed to the right of each lock profile name (located in the "Account List" area). To initiate or update the firmware information you must perform one of the following: Choose View Status of Lock (in the lock right click menu) or choose Update Status of All Locks from the Wireless Actions pull down menu.

 The Event log text is more user friendly:

Many lock audit trail event descriptions have been added or rewritten in order to clearly reflect each actual lock event.

 The Manually Unlock Door menu and functionality are improved:

Right click any "Linked" Networx lock profile; then choose Manually Unlock Door; The selectable times for the "Unlock this Door for" command have been reworked to be more user friendly.

 The Privacy termination by exit only option is supported by v5.5.3:

For Networx Privacy lock profiles; PDLN4100 and PDLN4500.

Use the Features screen, 4000 tab; Select "Privacy Duration" 251 to activate this feature.

 The Wireless Configuration names are customizable:

Located in the Wireless screen, this allows you to input custom names for each wireless config.

Click on the Edit button to rename; then click the Save button.

 The Transfer lock(s) to Another Gateway option is supported:

Located in the Gateway screen, on the Lock drop down menu. The Transfer lock(s) to Another Gateway option is available only during the Discover Locks function.